Business Data Networks and Security, 11e (Panko)

Chapter 1  Core Network Concepts and Terminology

1) Flooding a host with traffic so that it cannot serve its legitimate users is a ________ attack.

A) hacking

B) virus

C) malware

D) DoS

Answer:  D

Difficulty:  Basic

Question:  1a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

2) The KrebsOnSecurity.com attack ________.

A) was a denial of service attack.

B) used IoT devices to attack the site

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Basic

Question:  1b

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

3) Internet communication that does not involve a human is referred to with the term ________.

A) World Wide Web

B) impersonal

C) anonymous

D) IoT

Answer:  D

Difficulty:  Difficult

Question:  1c

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

4) If a host cannot reach a DNS server, it ________.

A) cannot have a host name

B) will have to rely on host names

C) will need to use a DHCP server

D) is unlikely to be able to communicate with most server hosts

Answer:  D

Difficulty:  Difficult

Question:  1d

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

5) The Mirai bots in the KrebsOnSecurity.com attack were ________.

A) client hosts

B) server hosts

C) IoT devices

D) all of the above

Answer:  C

Difficulty:  Basic

Question:  1e

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

6) ISPs ________.

A) carry traffic in the Internet

B) connect users to the Internet

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Basic

Question:  3a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

7) A packet sent from one host to another over the Internet will ALWAYS pass through ________.

A) a single ISP

B) more than 2 ISPs

C) exactly 2 ISPs

D) none of the above

Answer:  D

Difficulty:  Difficult

Question:  3c

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

8) We call any device connected to the Internet a(n) ________.

A) IP

B) client

C) router

D) host

Answer:  D

Difficulty:  Basic

Question:  4a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

9) Which of the following is a host when it's on a network?

A) a client PC

B) a server

C) a mobile phone

D) all of the above

Answer:  D

Difficulty:  Basic

Question:  4b

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

10) Which of the following is constantly leading to new applications?

A) increasing speed

B) increasing reliability

C) increasing availability wherever you are

D) all of the above

Answer:  D

Difficulty:  Basic

Question:  5a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

11) Who owns the Internet?

A) The U.S. Government

B) The United Nations

C) The IETF

D) No one

Answer:  D

Difficulty:  Basic

Question:  6a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

12) Who is in charge of the Internet?

A) the U.S. Government

B) the United Nations

C) the IETF

D) No one

Answer:  D

Difficulty:  Basic

Question:  6b

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

13) The IETF primarily ________.

A) manages the Internet

B) creates Internet standards

C) coordinates the work of ISPs

D) all of the above

Answer:  B

Difficulty:  Difficult

Question:  6c

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

14) From a security perspective, the Internet's ability to give access to nearly everyone is a ________.

A) good thing

B) bad thing

C) both A and B

D) neither A nor B

Answer:  B

Difficulty:  Difficult

Question:  7b

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology, Analytical Thinking

15) In the name Internet, "inter" means ________.

A) between

B) interior

C) international

D) interplanetary

Answer:  A

Difficulty:  Basic

Question:  8a

Objective:  Discuss how the Internet is changing and the security challenges these changes are creating.

AACSB:  Applying Information Technology

16) Depicting the Internet as a cloud symbolizes ________.

A) the Internet is amorphous

B) the Internet today is too complex to be understandable by humans

C) the Internet is too complex for anyone to depict

D) users do not have to know how the Internet operates

Answer:  D

Difficulty:  Basic

Question:  9a

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Analytical Thinking

17) When you use your tablet to access the internet, the tablet is a ________.

A) client host

B) server host

C) both A and B

D) neither A nor B

Answer:  A

Difficulty:  Basic

Question:  10a

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

18) Most servers are ________.

A) mainframes

B) Windows PCs

C) rack servers

D) routers

Answer:  C

Difficulty:  Basic

Question:  10b

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology

19) The client program is usually a browser.

Answer:  FALSE

Difficulty:  Basic

Question:  11b

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

20) The server program is always a webserver program.

Answer:  FALSE

Difficulty:  Basic

Question:  11c

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

21) The network stack implements the ________ layer process.

A) application

B) transport

C) physical

D) data link

Answer:  B

Difficulty:  Difficult

Question:  12a

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

22) The transport process always fragments the application message.

Answer:  FALSE

Difficulty:  Difficult

Question:  12c

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

23) What process adds a header in front of an application message or fragment?

A) application

B) transport

C) internet

D) data link

Answer:  B

Difficulty:  Basic

Question:  12d

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

24) A TCP message is called a ________.

A) datagram

B) packet

C) frame

D) none of the above

Answer:  D

Difficulty:  Basic

Question:  12e

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology

25) What process adds a header in front of a TCP segment?

A) Application

B) Transport

C) Internet

D) Data Link

Answer:  C

Difficulty:  Basic

Question:  12f

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

26) An IP packet may include a(n) ________.

A) application message or fragment

B) TCP segment or UDP datagram

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Difficult

Question:  13

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

27) When a frame arrives, the data link process on the destination host ________.

A) adds a header

B) removes a header

C) both A and B

D) neither A nor B

Answer:  B

Difficulty:  Basic

Question:  14a

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

28) The transport layer on the destination host ________.

A) reassembles the application message if it has been fragmented

B) removes the transport header

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Difficult

Question:  14b

Objective:  Explain basic concepts and terminology for the hosts (devices) that connect to the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

29) IPv4 addresses are ________ bits long.

A) 32

B) 64

C) 128

D) none of the above

Answer:  A

Difficulty:  Basic

Question:  15a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

30) ________ addresses are 32 bits long.

A) IPv4

B) IPv6

C) EUI-48

D) none of the above

Answer:  A

Difficulty:  Basic

Question:  15a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

31) Which is NOT in an IP packet?

A) Internet header

B) Transport header

C) Data link header

D) All of the above are part of the IP packet.

Answer:  C

Difficulty:  Difficult

Question:  16a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking

32) To deliver IPv4 packets, routers look at the ________ addresses.

A) dotted decimal notation

B) EUI-48

C) IPv4

D) all of the above

Answer:  C

Difficulty:  Basic

Question:  16c

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Application of Knowledge

33) Routing is based on a(n) ________.

A) IP address

B) single-network address

C) either A or B

D) neither A nor B

Answer:  A

Difficulty:  Basic

Question:  17a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Application of Knowledge

34) Router forwarding is called ________.

A) routing

B) switching

C) forwarding

D) none of the above

Answer:  A

Difficulty:  Difficult

Question:  17b

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

35) paths across a single network are called ________.

A) routes

B) data links

C) physical links

D) none of the above

Answer:  B

Difficulty:  Basic

Question:  18a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

36) When a source host sends a packet to a destination host, there will probably be ________ along the way.

A) more routes

B) more data links

C) about the same number of routes and data links

D) all of the above

Answer:  B

Difficulty:  Difficult

Question:  18d

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

37) If there are 10 routers between the source and destination hosts, how many transport processes will be involved on all devices?

A) 1

B) 2

C) 10

D) 12

Answer:  B

Difficulty:  Difficult

Question:  19a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

38) If there are 10 routers between the source and destination hosts, how may internet processes will be involved on all devices?

A) 1

B) 2

C) 10

D) 12

Answer:  D

Difficulty:  Difficult

Question:  19a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

39) Which of the following is an Internet supervisory protocol?

A) DNS

B) IP

C) both A and B

D) neither A nor B

Answer:  A

Difficulty:  Difficult

Question:  20

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

40) On the Internet, a ________ host usually gets a dynamic IP address.

A) client

B) server

C) both A and B

D) neither A nor B

Answer:  A

Difficulty:  Basic

Question:  21a

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

41) On the Internet, a ________ host usually gets a static IP address.

A) client

B) server

C) both A and B

D) neither A nor B

Answer:  B

Difficulty:  Basic

Question:  21b

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

42) A host with a(n) ________ IP address is easier to find than a host with a ________ IP address.

A) dynamic, static

B) static, dynamic

C) undetermined, static

D) undetermined, dynamic

Answer:  B

Difficulty:  Basic

Question:  21c

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

43) A DHCP server to assigns the client host the same IP address each time.

Answer:  FALSE

Difficulty:  Basic

Question:  21d

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

44) When Host A wishes to communicate with Host B, a DNS server provides the IP address of ________.

A) Host A

B) the DNS server

C) Host B

D) none of the above

Answer:  C

Difficulty:  Basic

Question:  22b

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

45) When Host A wishes to talk with Host B, the DHCP server provides the IP address of ________.

A) Host A

B) DNS server

C) Host B

D) all of the above

Answer:  A

Difficulty:  Difficult

Question:  22b

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

46) The originating host needs to contact the DNS server each time it sends a packet to the target host.

Answer:  FALSE

Difficulty:  Basic

Question:  22c

Objective:  Explain basic concepts and terminology for the Internet itself.

AACSB:  Applying Information Technology

47) In a single switched network, a ________ connects consecutive switches.

A) physical link

B) data link

C) route

D) none of the above.

Answer:  A

Difficulty:  Basic

Question:  23a

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

48) In a point-to-point single network, how many physical links will there be when a packet is transmitted?

A) 0

B) 1

C) 2

D) We cannot say with the information provided.

Answer:  B

Difficulty:  Basic

Question:  23b

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

49) When a packet is transmitted, there is a single data link in each ________.

A) point-to-point single network

B) switched network

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Basic

Question:  23c

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

50) In a point-to-point single network, there is always the same number of physical and data links.

Answer:  TRUE

Difficulty:  Difficult

Question:  23c

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

51) Which of the following is true?

A) Frames are carried inside packets.

B) Packets are carried inside frames.

C) both A and B

D) neither A nor B

Answer:  B

Difficulty:  Basic

Question:  25a

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology

52) A host sends a packet to another host containing seven single networks along the way. How many other hosts will there be along the route when Host A transmits?

A) 1

B) 2

C) 7

D) 9

Answer:  A

Difficulty:  Difficult

Question:  25b

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

53) A host sends a packet to another host containing six single networks along the way. How many data links are used for this transmission?

A) 1

B) 6

C) 7

D) 8

Answer:  B

Difficulty:  Difficult

Question:  25c

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

54) A host sends a packet to another host containing six single networks along the way. How many routes are included in this transmission?

A) 1

B) 6

C) 7

D) 8

Answer:  A

Difficulty:  Basic

Question:  25d

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

55) A host sends a packet to another host with six single networks along the way. How many frames are involved?

A) 1

B) 6

C) 7

D) 8

Answer:  B

Difficulty:  Basic

Question:  25e

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

56) A host sends a packet to another host with six single networks along the way. How many packets are involved in this transmission?

A) 1

B) 6

C) 7

D) 8

Answer:  A

Difficulty:  Basic

Question:  25d

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

57) In a single switched network with seven switches between a source and destination host, the source host addresses the frame ________.

A) to the first switch

B) to the destination host

C) to the first router

D) to the final switch

Answer:  B

Difficulty:  Difficult

Question:  25g

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

58) The final router in a route sends the packet being routed in a frame addressed to the ________.

A) next-hop router

B) destination host

C) local router

D) default router

Answer:  B

Difficulty:  Difficult

Question:  25h

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

59) All data link addresses are EUI-48 addresses.

Answer:  FALSE

Difficulty:  Basic

Question:  26a

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology

60) Source and destination IP addresses are found in the ________ header.

A) IP

B) Frame

C) TCP

D) all of the above.

Answer:  A

Difficulty:  Basic

Question:  26b

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

61) Source and destination EUI-48 addresses are found in ________ headers.

A) IP

B) frame

C) TCP

D) UDP

Answer:  B

Difficulty:  Basic

Question:  26c

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Application of Knowledge

62) Ethernet uses ________ addresses.

A) IPv4

B) IPv6

C) MAC

D) EUI-48

Answer:  D

Difficulty:  Basic

Question:  26d

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology

63) Wi-Fi uses ________ addresses.

A) IPv4

B) IPv6

C) MAC

D) EUI-48

Answer:  D

Difficulty:  Basic

Question:  26e

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology

64) Which of the following uses EUI-48 addresses?

A) Ethernet

B) Wi-Fi

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Basic

Question:  26e

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

65) Hosts on the Internet have ________ addresses.

A) IP

B) data link

C) both A and B

D) neither A nor B

Answer:  C

Difficulty:  Difficult

Question:  26f

Objective:  Explain basic concepts and terminology for single networks and their role on the Internet.

AACSB:  Applying Information Technology, Analytical Thinking, Application of Knowledge

66) ________ have access points.

A) Home access routers

B) Internet core routers

C) both A and B

D) neither A nor B

Answer:  A

Difficulty:  Basic

Question:  27a

Objective:  Explain the distinctions between Internet routers and personal access routers; explain the differences between personal access routers and wireless access points.

AACSB:  Applying Information Technology, Application of Knowledge

67) ________ have DHCP servers.

A) Home access routers

B) Internet core routers

C) both A and B

D) neither A nor B

Answer:  A

Difficulty:  Basic

Question:  27a

Objective:  Explain the distinctions between Internet routers and personal access routers; explain the differences between personal access routers and wireless access points.

AACSB:  Applying Information Technology, Application of Knowledge

68) Which of the following has better access point capabilities?

A) Home access routers

B) Corporate access points

C) Both have about the same access point capabilities.

Answer:  B

Difficulty:  Basic

Question:  27c

Objective:  Explain the distinctions between Internet routers and personal access routers; explain the differences between personal access routers and wireless access points.

AACSB:  Applying Information Technology, Application of Knowledge